UNCLASSIFIED

XR-7 Advanced Signal Processing Module

EMC Compliance Test Report

Document Number	RPT-XR7-2026-001

Revision	3.0

Issue Date	2026-06-18

Client	Vertex Aerospace Systems Ltd

Test Laboratory	Element Warwick — EMC & Wireless Laboratory

Accreditation	UKAS 17025 · Lab No. 0123

Standards applied:

•  EN 55032:2015 (CISPR 32) Class B

•  EN 55035:2017

•  IEC 61000-4-2

•  IEC 61000-4-3

Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-04-02 | T. Okafor | Initial issue |
| 2.0 | 2026-05-14 | T. Okafor | Added conducted-immunity results |
| 3.0 | 2026-06-18 | H. Nguyen | Full re-test after firmware v2.3; all sections re-issued |

Contents

•  1  Executive Summary

•  2  Equipment Under Test

•  3  Summary of Results

•  4  Radiated Emissions

•  5  Conducted Emissions

•  6  Radiated Immunity

•  7  Electrostatic Discharge Immunity

•  8  Appendix — Raw Data

•  9  Declaration of Conformity

1  Executive Summary

This report presents the electromagnetic compatibility (EMC) qualification results for the XR-7 Advanced Signal Processing Module (XR-7 Rev C, S/N SN-XR7-00417), tested at Element Warwick — EMC & Wireless Laboratory on behalf of Vertex Aerospace Systems Ltd.

Overall result: PASS — full compliance.

Testing was performed against the standards listed on the cover page. The sections that follow detail each discipline, the applicable limits, the measured results and the compliance margin. Full measurement extracts are provided in the appendix.

2  Equipment Under Test

The XR-7 is a rack-mounted advanced signal-processing module operating across 100 MHz to 6 GHz. It comprises a triple-channel RF front end, an FPGA processing core and a switched-mode DC supply.

| Attribute | Value |
| --- | --- |
| Model | XR-7 Rev C |
| Serial Number | SN-XR7-00417 |
| Power Supply | 48 V DC, 2.4 A max |
| Dimensions | 482 x 88 x 350 mm (19" 2U) |
| Operating Modes | Idle, Half-load, Full-load, Peak-burst |
| Cables / Ports | Shielded RF (SMA), CAT6A data, DC power harness |

3  Summary of Results

Table 3-1. Summary of EMC test results.

| Discipline | Standard / Limit | Key Result | Status |
| --- | --- | --- | --- |
| Radiated Emissions | EN 55032 Class B | Peak margin -6.2 dB @ 232 MHz | PASS |
| Conducted Emissions | EN 55032 Class B | Peak margin -4.8 dB @ 0.53 MHz | PASS |
| Radiated Immunity | IEC 61000-4-3, 10 V/m | Criterion A, all bands | PASS |
| ESD Immunity | IEC 61000-4-2, +/-8 kV | Criterion A, contact & air | PASS |
| Electrical Fast Transient | IEC 61000-4-4, +/-2 kV | Criterion A | PASS |

4  Radiated Emissions

Standard: EN 55032:2015 Class B    Site: 3 m Semi-Anechoic Chamber (SAC-2)    Detector: Quasi-peak / Average, CISPR 16-1-1 receiver

Radiated emissions were measured from 30 MHz to 6 GHz with the EUT in Full-load mode, the worst-case configuration identified during pre-scan. The chamber turntable was rotated 0-360 deg and the antenna height scanned 1-4 m at each azimuth to capture maximum emissions. The highest emission occurred at 232 MHz, 6.2 dB below the Class B limit.

Figure RE-1. Radiated emissions, horizontal polarisation, 30 MHz - 1 GHz vs EN 55032 Class B QP limit.

Table RE-1. Highest radiated emissions (worst-case azimuth/height).

| Freq (MHz) | Reading (dBuV/m) | Limit (dBuV/m) | Margin (dB) | Detector | Pol. | Status |
| --- | --- | --- | --- | --- | --- | --- |
| 156.00 | 33.8 | 40.0 | -6.2 | QP | H | PASS |
| 232.00 | 33.8 | 40.0 | -6.2 | QP | H | PASS |
| 2400.0 | 40.2 | 47.0 | -6.8 | AV | V | PASS |
| 1200.0 | 38.7 | 47.0 | -8.3 | AV | H | PASS |
| 312.50 | 31.2 | 40.0 | -8.8 | QP | V | PASS |
| 433.92 | 29.4 | 40.0 | -10.6 | QP | H | PASS |

5  Conducted Emissions

Standard: EN 55032:2015 Class B    Site: Shielded Room 1, 50 uH/50 Ohm LISN    Detector: Quasi-peak / Average, 150 kHz - 30 MHz

Conducted emissions on the AC mains port were measured across 150 kHz to 30 MHz using a two-line LISN. Both live and neutral conductors were scanned; the table lists the higher of the two. The worst-case margin was -4.8 dB at 530 kHz on the live conductor.

Figure CE-1. Conducted emissions on mains port (Live) vs EN 55032 Class B QP/AV limits.

Table CE-1. Highest conducted emissions.

| Freq (MHz) | QP (dBuV) | QP Limit | AV (dBuV) | AV Limit | Cond. | Status |
| --- | --- | --- | --- | --- | --- | --- |
| 0.53 | 61.2 | 66.0 | 48.3 | 56.0 | L | PASS |
| 0.80 | 58.9 | 63.5 | 45.1 | 53.5 | L | PASS |
| 1.50 | 55.4 | 60.0 | 42.7 | 50.0 | N | PASS |
| 3.20 | 52.1 | 56.0 | 39.8 | 46.0 | L | PASS |
| 6.40 | 49.7 | 56.0 | 37.2 | 46.0 | N | PASS |
| 12.8 | 47.3 | 60.0 | 35.6 | 50.0 | L | PASS |
| 24.0 | 44.8 | 60.0 | 33.1 | 50.0 | N | PASS |

6  Radiated Immunity

Standard: IEC 61000-4-3    Field: 10 V/m, 80% AM 1 kHz, 80 MHz - 6 GHz

The EUT was exposed to a 10 V/m field, 80% amplitude-modulated, across 80 MHz to 6 GHz on four faces in both polarisations. Signal integrity of all three RF channels was monitored continuously. No degradation beyond permitted limits was observed; performance criterion A was met throughout.

Table RS-1. Radiated immunity results by band and face.

| Band (MHz) | Field (V/m) | Pol. | Face | Observation | Criterion |
| --- | --- | --- | --- | --- | --- |
| 80 - 1000 | 10 | H | Front | No degradation | A |
| 80 - 1000 | 10 | V | Front | No degradation | A |
| 1000 - 2700 | 10 | H | Rear | No degradation | A |
| 2700 - 6000 | 10 | V | Side | No degradation | A |

7  Electrostatic Discharge Immunity

Standard: IEC 61000-4-2    Levels: Contact +/-4 kV, +/-8 kV; Air +/-8 kV, +/-15 kV

Electrostatic discharge was applied to all user-accessible points and to horizontal/vertical coupling planes. Ten discharges of each polarity were applied per point. The EUT continued to operate normally with no loss of function or data corruption; criterion A was achieved.

Table ESD-1. ESD immunity results.

| Discharge Point | Type | Level (kV) | Discharges | Observation | Criterion |
| --- | --- | --- | --- | --- | --- |
| Front panel / buttons | Contact | +/-4, +/-8 | 10 each | No effect | A |
| Chassis seams | Contact | +/-8 | 10 each | No effect | A |
| Ventilation grille | Air | +/-8, +/-15 | 10 each | No effect | A |
| HCP / VCP | Contact | +/-8 | 10 each | No effect | A |

8  Appendix — Raw Data

Table A-1. Full radiated-emissions sweep extract (worst azimuth).

| Test ID | Freq (MHz) | Reading | Limit | Margin | Detector | Status |
| --- | --- | --- | --- | --- | --- | --- |
| RE-1001 | 48.00 | 22.1 | 40.0 | -17.9 | QP | PASS |
| RE-1002 | 96.00 | 28.4 | 40.0 | -11.6 | QP | PASS |
| RE-1003 | 156.0 | 31.7 | 40.0 | -8.3 | QP | PASS |
| RE-1004 | 232.0 | 33.8 | 40.0 | -6.2 | QP | PASS |
| RE-1005 | 312.5 | 31.2 | 40.0 | -8.8 | QP | PASS |
| RE-1006 | 433.9 | 29.4 | 40.0 | -10.6 | QP | PASS |
| RE-1007 | 617.0 | 27.9 | 40.0 | -12.1 | QP | PASS |
| RE-1008 | 868.0 | 36.1 | 47.0 | -10.9 | QP | PASS |
| RE-1009 | 1200. | 38.7 | 47.0 | -8.3 | AV | PASS |
| RE-1010 | 1800. | 37.4 | 47.0 | -9.6 | AV | PASS |
| RE-1011 | 2400. | 40.2 | 47.0 | -6.8 | AV | PASS |
| RE-1012 | 3600. | 35.5 | 47.0 | -11.5 | AV | PASS |
| RE-1013 | 4800. | 33.9 | 47.0 | -13.1 | AV | PASS |
| RE-1014 | 6000. | 31.2 | 47.0 | -15.8 | AV | PASS |

9  Declaration of Conformity

On the basis of the tests described in this report, the XR-7 Advanced Signal Processing Module (XR-7 Rev C) is assessed as compliant with the requirements of the standards listed, for the configuration and operating modes tested.

Table 9-1. Approvals.

| Role | Name | Title | Date |
| --- | --- | --- | --- |
| Prepared by | H. Nguyen | EMC Test Engineer | 2026-06-18 |
| Reviewed by | T. Okafor | Senior EMC Engineer | 2026-06-18 |
| Approved by | D. Whitmore | Laboratory Manager | 2026-06-18 |